Project Onboarding & Architecture Guide

Project: RSU-Assisted Dual-Channel Imputation for IoV Trajectory Prediction in Mixed HDV/CAV Traffic

Welcome to the project! This guide will walk you through the architecture of our pipeline, the physical traffic dynamics we are modeling, and the engineering decisions behind our data generation and imputation strategies.

1. The Core Problem

Connected and Autonomous Vehicles (CAVs) broadcast their kinematic state (speed, position, acceleration) via V2X networks at 10 Hz. Our trajectory prediction system uses the last 3 seconds (30 steps) of this telemetry to forecast the next 5 seconds (50 steps). This forecast is critical for downstream applications like collision warnings and intelligent signal control.

The input for a single vehicle is a 30 × 7 matrix:

| Column | Feature | Source |
| --- | --- | --- |
| 0 | ego_speed | CAV's own V2X broadcast |
| 1 | ego_acceleration | CAV's own V2X broadcast |
| 2 | ego_x (position) | CAV's own V2X broadcast |
| 3 | ego_y (position) | CAV's own V2X broadcast |
| 4 | gap_to_leader | Leader vehicle's V2X broadcast (or radar) |
| 5 | leader_speed | Leader vehicle's V2X broadcast |
| 6 | follower_speed | Follower vehicle's V2X broadcast |

The Missing Data Challenge

In the near future, 70–90% of surrounding vehicles will be Human-Driven Vehicles (HDVs) with no V2X radio. This creates two distinct, highly challenging missing data patterns that standard interpolation methods cannot handle:

- Structural Feature Missingness (SFM): Affects columns 4, 5, and 6 across all 30 steps. HDVs cannot broadcast their speed. Because there is no past V2X data to interpolate from, standard imputers fail entirely.

- Temporal Cluster Missingness (TCM): Affects columns 0–3. V2X networks suffer from packet loss. Because packets are lost in bursts (due to fading or interference) rather than uniformly at random, we lose consecutive clusters of ego-state data.

To solve this, we are building a Dual-Channel Imputer:

- Channel 1 (Physics): Uses external roadside infrastructure (RSU cameras) and a Kalman Filter to track "dark" HDVs and resolve SFM.

- Channel 2 (Deep Learning): Uses a Physics-Constrained GRU-D neural network to reconstruct burst-missing ego-states and resolve TCM.

2. The Simulation Environment (Steps 1–3)

Real-world datasets do not contain perfect ground-truth states for disconnected HDVs. Therefore, we use the SUMO (Simulation of Urban MObility) engine to generate a perfect baseline. We later intentionally corrupt this baseline to simulate real-world sensor/network failures, allowing us to evaluate our models against the hidden ground truth.

The Highway Setup

- Configuration: A straight, 3-lane, 1 km highway. Speed limit is 33.33 m/s (120 km/h).

- Sampling Rate: 10 Hz (0.1s step length) to match physical V2X broadcast rates.

- RSU Zone: A virtual Roadside Unit (RSU) at the 500m mark tracks vehicles between 200m and 800m.

- Vehicle Types:

  - CAV: Uses CACC (Cooperative Adaptive Cruise Control). Min gap: 1.0m.

  - HDV: Uses Krauss (Stochastic human driving). Min gap: 2.5m.

Data Extraction & Penetration Sweeps

We extract data in two phases to avoid "looking into the future." First, we simulate and track active episodes. Once a vehicle leaves the RSU zone, its completed episode is sliced into non-overlapping 30-step inputs and overlapping 50-step trajectory labels.

Important Data Rule: Train/Test splits must always be done by simulation run or vehicle episode ID, never by random row index, to prevent data leakage between overlapping label windows.

We run automated sweeps across three CAV Market Penetration Rates (MPRs), keeping total traffic volume constant at 4,500 veh/hour:

- 10% CAV (Extreme missingness)

- 20% CAV (Base case)

- 30% CAV (High penetration)

To guarantee atomic, corruption-free alignment between inputs and labels, each simulation run is saved as a single compressed .npz archive containing X_gt, Y_gt, and metadata.

3. The Data Corruption Model (Steps 4–5)

We apply physical constraints to simulate real-world data loss without altering the ground truth (X_gt) arrays.

Structural Feature Missingness (SFM)

SFM distinguishes between two sensor modalities:

- Onboard Radar/LiDAR: Detects physical mass. Therefore, gap_to_leader (Col 4) is only masked if no vehicle physically exists ahead. If an HDV is ahead, the radar still sees it.

- V2X Telemetry: Requires a radio. Therefore, leader_speed (Col 5) and follower_speed (Col 6) are masked if the neighbor is an HDV or if no vehicle exists.

Temporal Cluster Missingness (TCM)

V2X packet delivery is atomic (all columns succeed or fail together) and packet loss is bursty. We model this using a Gilbert-Elliott two-state Markov chain:

- P_ENTER_BAD = 0.05

- P_EXIT_BAD = 0.40 (Mean burst length = 2.5 steps / 250ms).

TCM is applied directly on top of the SFM-corrupted window, accurately representing the chaotic reality of mixed traffic.

4. Key Discovery: The Platoon Phase Transition

While generating the datasets, we made a crucial scientific finding that challenges the standard linear assumptions in ITS literature.

- At 10% and 20% CAV: ~98% of ego-CAV leaders are HDVs.

- At 30% CAV: ~68% of ego-CAV leaders are other CAVs.

Why? The system undergoes a non-linear phase transition (percolation threshold). At 30%, CAV density reaches a critical point where vehicles cluster together to form self-sustaining CACC platoons (maintaining tight 1.0m gaps). Within a platoon, almost every vehicle tracks a CAV leader. This threshold effect dramatically shifts the V2X telemetry landscape and will be a major focal point in our final paper (Experiment E2).

5. Channel 1: The Physics Imputer (Steps 6–7)

To reconstruct the missing HDV leader speeds (SFM), we utilize simulated RSU cameras.

Step 6: RSU Camera Simulation

The RSU camera is a line-of-sight sensor that treats CAVs and HDVs as identical physical masses. We approximate real RSU hardware by applying published benchmark noise to the SUMO ground truth:

- Position Error: 0.5m std

- Speed Error: 0.3 m/s std

- Detection Rate: 95% (5% missed detection rate)

Note: Adding Gaussian noise to the absolute HDV position and subtracting the ego position is mathematically identical to adding noise directly to the relative gap. We apply noise directly to the relative gap column to maintain dimensional alignment.

Step 7: The 1D Kalman Filter

Feeding raw camera noise to a trajectory predictor harms performance. We implemented a 1D Constant-Velocity Kalman Filter (state: [x_leader, v_leader]) to smooth the RSU data.

- Missed Detections: The filter elegantly handles the 5% camera dropouts. During a dropout, it skips the update step but propagates the physical prediction forward, safely growing its covariance (uncertainty) without drifting.

- Ego Dead-Reckoning: During the filter's gap calculations, we use uncorrupted ego positions (X_gt[:,:,0:4]). This mirrors a real CAV's highly reliable onboard dead-reckoning (IMU/Odometry), which is immune to V2X dropouts.

- Performance: The filter successfully reduces the 0.3 m/s raw camera noise down to an incredibly accurate 0.197 m/s MAE against ground truth, imputing over 1.5 million cells.

6. Pipeline Architecture Summary

| Step | Input | Output | Used In |
| --- | --- | --- | --- |
| 1 — SUMO Setup | Initial config | .net.xml, .rou.xml, .sumocfg | All subsequent steps |
| 2 — Extraction | 20% CAV sim | X.npy, Y.npy, metadata.pkl | Debugging & Verification |
| 3 — Full Sweep | 3 MPRs × 20 seeds | 60 .npz files (Ground Truth) | Steps 4–11 |
| 4 — SFM Injection | X_gt, metadata | sfm_mask | Step 5 |
| 5 — TCM Injection | X_sfm, sfm_mask | X_corrupted, X_mask | Step 6 |
| 6 — RSU Simulation | X_gt from data_corrupted/ | rsu_obs (N, 30, 2) added to .npz | Step 7 (Channel 1) |
| 7 — Channel 1 (KF) | rsu_obs + Ego Dead-reckoning | X_ch1_imputed added to .npz | Step 9 (Integration) |
| 8 — Channel 2 (GRU-D) | X_corrupted (Ego bursts) | Trained Imputation Model | Step 9 (Integration) |

Next Up: Training the Physics-Constrained GRU-D for Channel 2!